MODULE -3

1. 802.11 PHY Layer Standards (Deep Explanation)

| Standard | Key Improvement | Deep Insight |
| --- | --- | --- |
| 802.11b | DSSS | Very robust but inefficient → large spreading reduces speed |
| 802.11a/g | OFDM | Introduced parallel transmission using subcarriers |
| 802.11n | MIMO | Multiple antennas → parallel spatial streams |
| 802.11ac | MU-MIMO + 256-QAM | Higher modulation → more bits per symbol |
| 802.11ax | OFDMA + 1024-QAM | Focus on efficiency in crowded networks |

Key Understanding:

- Earlier focus → raw speed

- Modern Wi-Fi → efficiency + multiple users

2. DSSS vs FHSS (More Intuition)

DSSS

- Each bit → converted into multiple bits using a chipping code

- Example:

  - 1 → 10110101

  - Even if noise corrupts some bits → receiver can recover original

Error-resilient but slow

FHSS

- Frequency keeps changing (hopping)

- Both sender & receiver follow same sequence

Avoid interference instead of correcting it

Comparison

| Feature | DSSS | FHSS |
| --- | --- | --- |
| Approach | Spread bits | Change frequency |
| Reliability | High | Moderate |
| Speed | Low | Low |
| Used in | 802.11b | Old Wi-Fi, Bluetooth |

3. Modulation (Very Important Concept)

Core Idea:

More bits per symbol = more speed BUT needs clean signal

Bit Mapping

| Modulation | Bits/Symbol |
| --- | --- |
| BPSK | 1 |
| QPSK | 2 |
| 16-QAM | 4 |
| 64-QAM | 6 |
| 256-QAM | 8 |
| 1024-QAM | 10 |

Why higher QAM needs high SNR?

Because:

- Constellation points become closer

- Noise can easily confuse symbols

So:

- Near AP → 1024-QAM

- Far away → fallback to BPSK

4. OFDM (Deep Concept)

Problem: Multipath interference

Signal arrives:

- Direct path

- Reflected path (walls, objects)

Causes ISI (Inter-Symbol Interference)

Solution: OFDM

Instead of:

- 1 fast stream

Use:

- Many slow streams

How it works:

- Channel split into subcarriers

- Each carries part of data

- All are orthogonal (no interference)

Key Benefit:

- Handles reflections very well

- High spectral efficiency

5. Frequency Bands (Deep Insight)

2.4 GHz

- Long range

- Penetrates walls

- Crowded (Wi-Fi + Bluetooth + microwave)

Only 3 clean channels (1,6,11)

5 GHz

- Shorter range

- More channels

- Supports:

  - 40 MHz

  - 80 MHz

  - 160 MHz

Higher bandwidth = higher speed

6 GHz (Wi-Fi 6E)

- Almost no interference

- Huge spectrum

Best for:

- High-density environments

- Future networks

6. Guard Interval (GI)

Problem:

Reflected signals arrive late → overlap next symbol

Solution:

Insert small gap between symbols

Types:

| GI | Duration | Use |
| --- | --- | --- |
| Long | 800 ns | Safe |
| Short | 400 ns | Faster |

Short GI increases throughput because:

- Less idle time

BUT:

- Requires clean environment

7. PHY Frame Structure (Detailed)

PPDU Components:

1. Preamble

- Synchronization

- Channel estimation

Receiver "locks" to signal

2. PLCP Header

Contains:

- Modulation type

- Data rate

- Length

Helps receiver decode payload

3. PSDU (Payload)

- Actual data from MAC layer

8. OFDM vs OFDMA (Very Important for Wi-Fi 6)

OFDM

- One user at a time

- Entire channel used

OFDMA

- Channel split into Resource Units (RU)

Multiple users transmit simultaneously

Example:

- 20 MHz channel

- Split into 4 users

Each gets small portion

Benefit:

- Less latency

- Better efficiency

9. MIMO vs MU-MIMO

MIMO

- Multiple antennas

- One user at a time

Speed increases

MU-MIMO

- Multiple users simultaneously

Capacity increases

Difference:

| Feature | MIMO | MU-MIMO |
| --- | --- | --- |
| Users | 1 | Many |
| Goal | Speed | Capacity |

10. PPDU, PLCP, PMD (Clarified)

PLCP

- Interface between MAC & PHY

- Builds header

PMD

- Physical transmission

- Converts bits → radio waves

PPDU

Final transmitted frame

11. PPDU Types Evolution

| Type | Used in |
| --- | --- |
| Legacy | 802.11a/g |
| HT | 802.11n |
| VHT | 802.11ac |
| HE | 802.11ax |

HE (Wi-Fi 6) Special Feature:

- Trigger-based communication

- Scheduled uplink

AP controls traffic → less collision

12. Data Rate Formula (Core Concept)

Data Rate=Nsc×Bps×RTs

Explanation:

| Term | Meaning |
| --- | --- |
| Nsc | Number of subcarriers |
| Bps | Bits per symbol (modulation) |
| R | Coding rate |
| Ts | Symbol duration |

Extended Real Formula:

Data Rate=Subcarriers×Bits×Coding Rate×Spatial Streams÷Symbol Time

Example Insight:

To increase speed:

- Increase channel width → more subcarriers

- Increase modulation → more bits

- Increase spatial streams

- Reduce GI

Final Concept (Very Important)

Wi-Fi performance depends on:

- Distance from AP

- Interference

- Channel width

- Modulation

- Number of users