1. What are the pillars of Wi-Fi security?

The three core pillars are:

- Authentication — Verifying the identity of a device/user before granting network access (e.g., passwords, certificates, 802.1X).

- Encryption — Scrambling data in transit so it can't be read by unauthorized parties (e.g., AES, TKIP).

- Integrity — Ensuring data is not tampered with during transmission (e.g., MIC — Message Integrity Code in WPA2).

Some frameworks also add Access Control (controlling what authenticated users can do) and Availability (protecting the network from denial-of-service attacks) as additional pillars.

2. Explain the difference between authentication and encryption in WiFi security.

|  | Authentication | Encryption |
| --- | --- | --- |
| Purpose | Proves who you are | Protects what you send |
| When | Before joining the network | After joining, during data transfer |
| Example | Entering a WiFi password, 802.1X login | AES encrypting your packets |
| Analogy | Showing ID at the door | Speaking in a secret code inside |

Authentication happens first — only after a device is authenticated does encryption of traffic begin. Both are necessary; authentication alone doesn't protect data in transit, and encryption alone doesn't prevent unauthorized users from joining.

3. Explain the differences between WEP, WPA, WPA2, and WPA3.

| Feature | WEP | WPA | WPA2 | WPA3 |
| --- | --- | --- | --- | --- |
| Year | 1997 | 2003 | 2004 | 2018 |
| Encryption | RC4 (40/104-bit) | RC4 + TKIP | AES-CCMP | AES-GCMP-256 |
| Key Management | Static key | TKIP (dynamic) | CCMP | SAE (Dragonfly) |
| Integrity | CRC-32 (weak) | MIC | MIC (stronger) | BIP-GMAC-256 |
| Security Level | Broken/Obsolete | Weak | Strong | Very Strong |
| Vulnerability | IV reuse, crackable in minutes | TKIP vulnerabilities | KRACK attack (patched) | Side-channel attacks (minor) |

- WEP — First standard, fundamentally broken due to weak IV (Initialization Vector) implementation.

- WPA — Temporary fix using TKIP, backward compatible with WEP hardware but still has weaknesses.

- WPA2 — Introduced AES encryption, became the long-standing industry standard.

- WPA3 — Current standard, uses SAE for stronger password-based authentication and forward secrecy.

4. Why is WEP considered insecure compared to WPA2 or WPA3?

WEP is insecure for several critical reasons:

- Weak IV (Initialization Vector): WEP uses a 24-bit IV with RC4. Because the IV is short and reused frequently, attackers can collect enough packets to statistically crack the key — in as little as a few minutes using tools like Aircrack-ng.

- Static encryption key: Everyone on the network shares the same key with no rotation mechanism.

- Weak integrity check: Uses CRC-32, which is not cryptographically secure — an attacker can modify packets without detection.

- No mutual authentication: Only the client authenticates to the AP, not the other way around, enabling rogue AP attacks.

WPA2 addresses all of these with AES-CCMP encryption, dynamic per-session keys, and a cryptographic MIC. WPA3 goes further with SAE, which prevents offline dictionary attacks even if someone captures the handshake.

5. Why was WPA2 introduced?

WPA2 was introduced in 2004 because:

- WEP was completely broken — tools could crack WEP keys in under 5 minutes.

- WPA was a stopgap — WPA used TKIP which still relied on RC4 underneath and had known vulnerabilities (e.g., TKIP MIC attack).

- Need for stronger encryption — WPA2 mandated AES-CCMP (Counter Mode with CBC-MAC Protocol), a far more robust algorithm.

- IEEE 802.11i standard — WPA2 was the full implementation of the 802.11i security amendment, whereas WPA was only a partial implementation to quickly replace WEP.

WPA2 became mandatory for WiFi certified devices from 2006 onwards and remained the standard for over a decade.

6. What is the role of the Pairwise Master Key (PMK) in the 4-way handshake?

The PMK is the foundation of the 4-way handshake:

- It is derived from the passphrase + SSID (in WPA2-Personal) using PBKDF2, or provided directly by the RADIUS server (in WPA2-Enterprise).

- The PMK itself is never transmitted over the air — this is critical for security.

- Instead, both the client (supplicant) and the AP (authenticator) independently derive the PTK (Pairwise Transient Key) from the PMK using:

  - PMK + ANonce (AP's random number) + SNonce (client's random number) + MAC addresses of both devices

- The PTK is then used to encrypt actual data traffic.

So the PMK acts as the shared secret that proves both sides know the password, without ever exposing it over the wireless medium.

7. How does the 4-way handshake ensure mutual authentication between the client and the access point?

The 4-way handshake works as follows:

AP  ──── Message 1 (ANonce) ────────────────►  Client

AP  ◄─── Message 2 (SNonce + MIC) ──────────  Client

AP  ──── Message 3 (GTK + MIC, install PTK) ► Client

AP  ◄─── Message 4 (ACK) ───────────────────  Client

Mutual authentication is achieved because:

- The client proves it knows the PMK by computing a valid MIC (Message Integrity Code) in Message 2 using the derived PTK.

- The AP proves it knows the PMK by computing a valid MIC in Message 3.

- If either MIC fails verification, the handshake is aborted — meaning a rogue AP without the correct PMK cannot complete the handshake.

- The nonces (ANonce, SNonce) ensure the session keys are fresh and unique — preventing replay attacks.

This two-way verification is what makes WPA2 mutually authenticating.

8. What will happen if we put a wrong passphrase during a 4-way handshake?

When a wrong passphrase is entered:

- The client derives an incorrect PMK from the wrong passphrase.

- It generates an incorrect PTK using that bad PMK.

- In Message 2, the client sends a MIC computed with the wrong PTK.

- The AP computes the expected MIC using the correct PTK — the MICs don't match.

- The AP silently drops the handshake (no error is sent back for security reasons).

- The client retries the handshake — typically up to 4 times.

- After repeated failures, the client is denied association and the connection fails.

From the user's perspective, they simply see "Incorrect Password" or "Unable to connect." In a Wireshark capture, you would see repeated 4-way handshake attempts with no progression to data frames.

9. What problem does 802.1X solve in a network?

802.1X solves the problem of unauthenticated network access — specifically the weakness of shared passwords (PSK).

Problems with PSK (Pre-Shared Key):

- One password for all users — if it leaks, everyone is compromised.

- No individual user accountability.

- Revoking access requires changing the password for everyone.

802.1X introduces port-based Network Access Control (NAC):

- Each user authenticates with individual credentials (username/password, certificate, smart card).

- Uses a RADIUS server as the authentication backend.

- The AP acts as an Authenticator — it does not grant access until the RADIUS server approves.

- Access can be revoked per user without affecting others.

- Enables VLAN assignment per user based on identity.

It is essential in enterprise environments where hundreds of users need individual, auditable, and revocable access.

10. How does 802.1X enhance security over wireless networks?

802.1X enhances wireless security in multiple ways:

- Per-user authentication: Each device/user has unique credentials — no shared passwords.

- Dynamic encryption keys: After successful authentication, unique per-session PTKs are generated for each user, so traffic cannot be decrypted by other users on the same network.

- Centralized access control: The RADIUS server enforces policies — who can connect, when, and to which VLAN.

- EAP framework flexibility: Supports many authentication methods (EAP-TLS with certificates, PEAP with passwords, EAP-TTLS, etc.), allowing strong mutual authentication.

- Rogue AP protection: With EAP-TLS, the server presents a certificate — clients can verify they are connecting to the legitimate server, not a rogue AP.

- Audit trail: Every authentication event is logged on the RADIUS server, providing full accountability.

- Seamless revocation: Disabling one user account on the RADIUS server immediately cuts off that user's access without impacting others.

.