MODULE -2

1. SplitMAC Architecture and AP Performance

Concept Overview

In a traditional Autonomous Access Point architecture, each AP independently performs all IEEE 802.11 MAC layer functions, including both real-time packet handling and higher-level management decisions.

In contrast, SplitMAC architecture (used in controller-based deployments like Cisco Unified Wireless) divides these responsibilities between the Access Point (AP) and the Wireless LAN Controller (WLC).

Functional Distribution

AP Responsibilities (Time-Critical / Real-Time)

- Beacon frame generation

- Probe request/response handling

- Frame acknowledgment (ACK)

- Encryption/Decryption (AES, TKIP)

- Signal strength measurement (RSSI)

- Forwarding of client data frames

These tasks require low latency and immediate processing, hence they remain on the AP.

WLC Responsibilities (Centralized Management)

- Authentication (802.1X, EAP)

- Key management (PMK, PTK derivation)

- Radio Resource Management (RRM)

- QoS enforcement

- Security policies (ACLs, WIPS)

- Inter-subnet roaming (Layer 3 mobility)

Performance Impact

- Reduced AP CPU Utilization:
AP hardware is optimized for packet forwarding rather than control-plane decisions.

- Centralized Intelligence:
The WLC has a global view of the network, enabling:

  - Dynamic channel allocation

  - Transmit power optimization

  - Seamless roaming across APs

- Scalability:
Hundreds or thousands of APs can be managed efficiently.

2. CAPWAP Protocol and Operational Flow

Definition

CAPWAP (Control and Provisioning of Wireless Access Points) is an IETF standard protocol (RFC 5415) used for communication between Lightweight APs and WLCs.

Operational Workflow

1. Discovery Phase

The AP identifies the WLC using:

- Layer 2 broadcast

- DHCP Option 43

- DNS resolution (CISCO-CAPWAP-CONTROLLER)

- Static configuration

2. DTLS Establishment

- A DTLS (Datagram TLS) tunnel is created

- Ensures encryption and integrity of control traffic

3. Join Phase

- AP sends a Join Request

- WLC verifies AP identity using X.509 certificates

- Prevents unauthorized APs from joining

4. Image Download & Configuration

- Firmware version comparison

- Automatic upgrade/downgrade if needed

- AP downloads:

  - SSIDs

  - VLAN mappings

  - Security settings

5. Run State

- AP becomes operational

- Starts beaconing SSIDs

- Begins forwarding client traffic

3. CAPWAP in OSI Model and Tunnel Types

OSI Layer Placement

- CAPWAP operates at Layer 7 (Application Layer)

- Uses UDP (Layer 4) for transport

CAPWAP Tunnels

Control Tunnel (UDP Port 5246)

- Used for:

  - Configuration

  - Monitoring

  - Authentication

- Always encrypted using DTLS

Data Tunnel (UDP Port 5247)

- Encapsulates:

  - 802.11 frames → 802.3 Ethernet frames

- Forwards client traffic to WLC

- Encryption is optional

  - Disabled in most deployments for performance reasons

4. Lightweight vs Cloud-Based Access Points

Lightweight APs (Controller-Based)

- Require on-premise WLC (physical or virtual)

- Control plane is local

Advantages:

- Low latency

- Deep packet inspection

- Advanced enterprise features

Use Case: Large campuses, enterprises

Cloud-Based APs (e.g., Cisco Meraki)

- Managed via cloud dashboard

- No on-site WLC required

Advantages:

- Zero-touch deployment

- Centralized cloud management

- High scalability

Limitations:

- Dependency on internet for management

- Less granular control compared to WLC

5. CAPWAP Tunnel Maintenance (Heartbeats)

Mechanism

- AP sends Echo Requests every ~30 seconds

- WLC responds with Echo Replies

Failure Detection

- If multiple responses are missed:

  - Tunnel is considered down

  - AP enters Discovery Mode

Outcome

- AP attempts to:

  - Reconnect to primary WLC

  - Discover backup controllers

6. Sniffer Mode vs Monitor Mode

Sniffer Mode

- AP captures raw 802.11 frames

- Forwards traffic to tools like Wireshark

Purpose:

- Deep packet analysis

- Troubleshooting connectivity issues

Monitor Mode

- AP acts as a dedicated security sensor

- Does NOT serve clients

Functions:

- Rogue AP detection

- Intrusion detection (WIDS/WIPS)

- DoS attack identification

7. WLC over WAN: FlexConnect Mode

Concept

FlexConnect is designed for remote branch deployments where the WLC is across a WAN.

Operation Modes

Local Switching

- Client traffic is bridged locally

- Does NOT traverse WAN

Central Authentication

- Authentication still handled by WLC

Survivability

- If WAN fails:

  - AP continues operation

  - Can authenticate clients locally (if configured)

  - Maintains business continuity

8. Challenges of Deploying 50+ Autonomous APs

Operational Issues

1. Management Complexity

- Manual configuration per AP

- No centralized control

2. RF Interference

- No coordination between APs

- Leads to:

  - Co-channel interference

  - Signal overlap

  - Performance degradation

3. Poor Roaming Experience

- No fast roaming protocols

- Clients exhibit sticky behavior

- Leads to:

  - Packet loss

  - Voice/video disruption

4. Lack of Visibility

- No centralized monitoring

- Difficult troubleshooting

9. WLC Failure in Local Mode

Behavior in Local Mode

- AP is fully dependent on WLC

- All client data flows through WLC

Failure Scenario

When WLC becomes unavailable:

- CAPWAP tunnels are terminated

- AP loses configuration context

- SSIDs stop broadcasting

Impact

- Immediate client disconnection

- Network outage

Recovery Mechanism

- AP enters Discovery Mode

- Attempts to connect to backup WLC

Mitigation Strategy

- Use High Availability (HA) WLCs

  - Active/Standby controllers

  - Stateful failover

  - Minimal disruption